/** \page BasicControlPage Basic controller example: PushupController

This section describes a more complicated module: a RHex-specific

controller which controls the robot to continuously do ``pushups''. It

demonstrates the simplest way of building a controller in RHexLib, how

basic motor control tasks are accomplished. It also shows some of the

configuration facilities commonly used in RHexLib.

This controller assumes that RHex is in the standing position to start

with. Starting there, it brings all the legs to a vertical position

(making the robot sit), waits there for a moment, and then stands up

again, restarting the cycle.

In this section, we present the PushupModule implementation in

isolation. Unlike our previous example, a complete implementation of

an example executable requires many other RHex specific components

such as calibration tools which are not available in the core

RHexLib. In any case, the source code associated with this tutorial

can be found under the examples directory.

\section moddefsec The Module Definition

Here we describe the details of how to define the  PushupController

module class and explain some of the components required by this

controller. Usually, the definitions of module classes in RHexLib are done

in separate header files, defining the interface of the module with the rest

of the library. In this case, the class definition will reside in

PushupController.hh.

\code

#include "ModuleManager.hh"

#include "PositionControl.hh"

#include "Profiler.hh"

class PushupController : public Module {

public:

PushupController ( void )

: Module( "pushupcontroller", 0, true, false ) { };

// Module base class interface

void  init ( void );

void  uninit ( void ) { };

void  activate ( void );

void  deactivate ( void );

void  update ( void );

\endcode

This part of the class definition is straightforward. We simply define

the standard module functions in a way similar to the previous

example, the PrintModule. Once again, the name and index of the module

are determined in the definition of the class constructor, through the

instatiation of the base class Module. Note also that in this case,

all the module functions except the uninit() method will be defined in

the implementation file PushupController.cc.

Next we define a few private variables nad methods, local to

PushupController.

\code

private:

PositionControl *control[6];

\endcode

These member variables will be pointers to the six different position

controller modules, associated with each one of the six legs.

PositionControl modules implement basic proportional derivative(PD) control

feedback for all the motors in the system. Through their public interface,

our controller will be able to set the target positions for each leg,

updated every millisecond, commanding the legs perform the desired actions.

\code

MotorGains_t     oldGains[6];

\endcode

These member variables hold the PD gains that may have been in use by the

position controllers before the pushup controller is activated. When the

controller is deactivated, it will restore these gains to leave the position

controllers in their previous state. This is usually a good practice because

other components in the system might be expecting the PD gains to remain

unchanged for proper operation.

\code

Profiler         legProfiler;

\endcode

This member variable instantiates a Profiler object (see Profiler.hh

in the examples directory), which is responsible from generating the

leg position as a function of time, also called leg profile. Note that

we only have one such object because, for the pushup controller, all

the legs are synchronized. Cconsequently, the desired positions for

all six legs will be the same at all times.

In RHexLib, the actual PD control of the leg motors and the generation

of the associated position profile are performed separately. Every

millisecond, our controller will ask the leg profiler the desired

position for all the legs. Then, it will set this new target on all

six position controllers. This scheme abstracts the generation of the

desired leg trajectories from the local PD feedback loop on the leg

motors.

\code

void trackCurrentProfile( void );

\endcode

This utility method provides a facility to read the current value of the leg

profiler and inform all the motor position controllers about the new target.

\code

double lowerTime;  // Time in seconds to lower the robot

double chillTime;  // Time in seconds to chill, sitting down

double standTime;  // Time in seconds to raise the robot up

double waitTime;   // Time in seconds to wait standing up.

\endcode

These member variables determine how much time the controller should spend

in each of its stages. The pushup behavior can be decomposed into four

stages: lowering, chilling after sitting down, standing up and waiting in a

standing position. These variables determine how much time is spent in each

of these stages.

\code

int phase;

\endcode

This member variable indicates the current discrete ``phase'' of the

controllers. In implementing the aforementioned four different stages, the

pushup controller goes through six phases, where the controller actions are

defined as follows:

\li 0 : Starting to lower the robot

\li 1 : Waiting for the robot to lower

\li 2 : Wait for a while in the sitting position

\li 3 : Starting to standup

\li 4 : Waiting for the robot to standup

\li 5 : Wait for a while in the standing position

Another way to write this controller is to use the StateMachine

facilites of RHexLib. For this relatively simple example, we do not

need the power of state machines. If there were many more than six

states, or if the states were not visited in a simple cycle, or if the

events that trigger state transitions are were not so simple, state

machines would probably make the task simpler.  State Machines are

covered in later tutorials about a \ref ToyMachinePage "toy

controller" and a \ref SupMachinePage "supervisory controller".

\code

double mark;

};

\endcode

This member variable is used to hold the state time of each phase in order

to switch to the next phase at appropriate times, determined by the values

of the timing member variables described above.

\section initmethodsec The  init Method

There are two tasks usually performed by the init method of modules:

1) locate other modules that are going to be used; and 2) initialize

member variables by reading configuration information from a resource

file..

With the function  MMFindModule(), RHexLib provides a means to acquire

a pointer to other module objects in the system. Almost all the

functionality of the library is implemented with modules, including low

level motor control, higher level behaviors such as walking, standing,

sitting, pronking as well as utilities such as logging data. Normally, in

main(), all such modules to be used by an executable program are

``added'' to the system through calls to MMAddModule(). The module

manager thus has a list of all these components, and can find modules by

their name and index, which are two of the parameters to the

MMFindModule() function.

\code

void PushupController::init( void ) {

int i;

for ( i = 0; i < 6; i ++ ) {

if ( ( control[i] = ( PositionControl * )

MMFindModule( POSITIONCONTROL_NAME, i )) == NULL)

MMFatalError ( "PushupController::init",

"Cannot find motor controller!" );

}

\endcode

This part of the init method locates all six PositionControl

modules. All of these six modules have the same name, defined in

StdModules.hh as the macro POSITIONCONTROL_NAME (defined in

PositionControl.hh), and their index identifies which motor axis they

correspond to. If MMFindModule() function call returns NULL,

indicating that the required module has not yet been added to the

module manager's list, a fatal error is generated, and the program

exits.

\code

lowerTime = MMGetFloatSymbol( "pushup_lower_time" , 1.0 );

chillTime = MMGetFloatSymbol( "pushup_chill_time" , 0.3 );

standTime = MMGetFloatSymbol( "pushup_stand_time" , 0.4 );

waitTime = MMGetFloatSymbol( "pushup_wait_time" , 0.3 );

}

\endcode

The second part of the init method initializes the timing

configuration of the pushup machine. RHexLib maintains a global symbol

table, where symbol names are associated with either a floating point

or a string value. Modules in the system can access the table using

MMGetFloatSymbol() and MMGetStringSymbol() to retrieve the value of a

particular symbol. These functions also take a default value as an

argument, to be returned if the requested symbol cannot be found.

Usually, the symbol table is initialized by loading a ``configuration

file'' using the MMReadConfig() function in main(). This configuration

file includes statements such as:

\code

pushup_lower_time = 0.9;

pushup_chill_time = 0.2;

pushup_stand_time = 0.5;

pushup_wait_time = 0.2;

\endcode

which assign values to symbols. This mechanism provides means of

reconfiguring the behavior of controllers and modules without requiring that

the executable be recompiled.

\section actmetsec The activate Method

The only modules used by the pushup controller are the motor position

controllers. Moreover, they are required throughout the whole operation of

the controller. As a consequence, the activate module method in this case is

responsible for requesting exclusive access to all six position controllers

and adjusting the PD gains appropriately.

\code

void PushupController::activate( void ) {

int          i;

MotorGains_t gains;

phase = 0;

\endcode

Every time the pushup controller is activated, it will start from phase 0:

starting to lower. The phase must be initialized in the activate method

because the same instantiation of this module may be activated more than

once (at different times) by different supervisory controllers.

\code

for ( i = 0; i < 6; i++ )

MMGrabModule( control[i], this );

\endcode

This statement requests exclusive access to the motor position

controller modules whose pointers were acquired in the init

method. The functions MMGrabModule() and MMReleaseModule() are thus

used in pairs to acquire and release the exclusive use of other

modules in the system. They should be used whenever the functionality

of a module is needed to make sure that the operation of other

components in the system which might be using the same modules is not

disturbed.

In this context, there are two types of modules: SINGLE_USER and

MULTI_USER. The type of a module is determined by the constructor of

the Module base class. The behavior of MMGrabModule() is different for

these two types.

Single user modules can be grabbed by only one other module at a time. If a

second grab request is received, the system gives a fatal error and

exits. This mechanism makes sure that, for example, the motor position

controllers are not used by more than one entity at a time, which could

result in pretty chaotic behavior. Multi user modules can be grabbed as many

times as necessary.

Another feature of MMGrabModule() is that the grabbed module is also

activated if it is not already active. Similarly, the

MMReleaseModule() function deactivates the module if it has been

released as many times as it has been grabbed.  Hence, for multi user

modules, the first call to the grab function activates the module and

the last call to the release function deactivates the module.

\code

gains.kp = 5.0;

gains.kd = 0.14;

for ( i = 0; i < 6; i++ ) {

control[i]->getGains( & oldGains[i] );

control[i]->setGains( & gains );

}

}

\endcode

This code segment saves the PD gains of the motor controllers and sets the

new gains to appropriate values for the pushup behavior.

\section The deactivate Method

The deactivate module method is usually responsible for releasing the

modules that have been grabbed in the activate method. In addition, it

usually restores their states to what they were before they were grabbed.

\code

void PushupController::deactivate( void ) {

int i;

for ( i = 0; i < 6; i++ )

control[i]->setGains( & oldGains[i] );

\endcode

This code segment restores the PD gains of the motor position controllers to

their old values.

\code

for ( i = 0; i < 6; i++ )

MMReleaseModule( control[i], this );

}

\endcode

Finally, all six motor position controllers are released to grant access to

anybody else who wants to use them.

\section updmetsec The update Method

The update module method is where the bulk of the work is done for the

controller. All the other module methods are mainly for initialization and

clean integration with the rest of the system and do not much else.

Depending on the current phase of the controller, the update method, being

called once every millisecond, has to perform different tasks. As a

consequence, it ends up being a big case statement, implementing a primitive

state machine which linearly proceeds through 6 states (or phases). In later

sections, we will see more structured and formal ways of defining state

machines, which will allow the programmer to define much more complex

behaviors. Beyond 5 or 6 states, with transitions depending on events other

than simple timers, defining state machines through case statements quickly

becomes unelegant.

\code

void PushupController::update( void ) {

int           i;

MotorTarget_t target;

double        now = MMReadTime();

\endcode

This segment defines the local variables. An important function to note is

MMReadTime, which returns the current time in seconds.

\code

switch ( phase ) {

case 0:   // Starting to lower the robot

legProfiler.setup( now, now + lowerTime, 0, M_PI / 2 );

\endcode

As described in the preceding sections, phase 0 in the pushup controller is

when the robot starts to lower. Hence, in this state, we initialize the leg

profile to bring the leg to a horizontal position and immediately proceed

with the next phase.

The  setup(t1, t2, val2, val2) method of the  Profiler class is

used to initialize a linear time trajectory for the leg between two angle

values. The resulting trajectory has the following functional form:

<CENTER>

\f$

f(t) = \left\{\begin{array}{l@{\hspace{0.3in}}l}

val_1 & t < t_1 \\

val_1 + {(t-t_1) (val_2-val_1)}/{t_2-t_1)} & t_1 \leq t \leq t_2 \\

val_2 & t > t_2 \\

\end{array}\right.

\f$

</CENTER>

Note that this interface does not know anything about the \f$2 \pi\f$

periodicity of angles, and interprets \f$val_1\f$ and \f$val_2\f$ as

real numbers. The pushup controller, in order to lower the robot to

the ground, sets up a linear profile starting from 0 (vertical legs)

and going to \f$\pi/2\f$ (horizontal legs) in the amount of time

configured in the init method.

\code

mark = now;

phase = 1;

break;

\endcode

This phase of execution also marks the current time to measure how much more

the controller needs to wait before initiating the chilling phase. Also, the

assignment to the phase member variable transitions into phase 1, which

waits for the robot lowering to end.

\code

case 1:   // Waiting for the robot to lower

if ( now < ( mark + lowerTime ) )

trackCurrentProfile();

else {

mark = now;

phase = 2;

}

break;

\endcode

During phase 1, the controller does not change the profile which has been

setup in phase 0. However, it reads the current target position from the

profile and commands the motors to go to the current target angle through

repeated calls to the  trackCurrentProfile method (whose details are

explained later). When the legs reach their horizontal position

(i.e. when  lowerTime amount of time has elapsed), the controller

switches to phase 2.

\code

case 2:   // Wait for a while in the sitting position

if ( now > ( mark + chillTime ) ) {

mark = now;

phase = 3;

}

break;

\endcode

During this phase, the controller simply waits for a configured amount of

time, without commanding new targets for the legs. Due to the fact that the

position control modules retain their latest assigned target, the legs

maintain their horizontal positions, chilling in a sitting down

position. Once again, note the use of the  mark member variable, which

is a typical way of handling time-based transitions between different

controller phases.

\code

case 3:   // Starting to standup

legProfiler.setup( now, now + standTime, M_PI / 2, 0 );

mark = now;

phase = 4;

break;

\endcode

Similar to phase 0, our controller next sets up a new leg trajectory

in this phase. This time, however, the legs are commanded to go from

\f$\pi / 2\f$ to \f$0\f$, effectively raising the robot to a standing

position.

\code

case 4:   // Waiting for the robot to standup

if ( now < ( mark + standTime ) )

trackCurrentProfile();

else {

mark = now;

phase = 5;

}

break;

case 5:   // Wait for a while in the standing position

if ( now > ( mark + waitTime ) ) {

mark = now;

phase = 0;

}

break;

}

}

\endcode

Phases 4 and 5 perform exactly the same functionality as the phases 1

and 2, first tracking the leg profiles for standing up, and then

waiting a little while in a standing position. Note that the

controller returns to phase 0 once done with phase 5, restarting the

sequence by lowering the robot to the ground.

\section trprsec The trackCurrentProfile Method

As explained in the preceding sections, the commonly performed task of

reading the desired positions for the legs from the leg profiler and

setting the targets of the position controllers is done by this

method.

\code

void PushupController::trackCurrentProfile ( void ) {

int i;

MotorTarget_t target;

target = legProfiler.getVal();

// Command all 6 motor controllers to go to that position.

for ( i = 0; i < 6; i++ )

control[i]->setTarget ( & target );

}

\endcode

The data type  MotorTarget_t holds the desired position, velocity and

acceleration for the motor, which is used by the local PD feedback

control. The leg profiler extracts this information from the functional form

given in the previous section, and returns it through its  getVal()

method. This target can then be directly passed into

PositionControl::setTarget().

*/