/** \page BasicModulePage  Basic module example: PrintModule

This section describes the details of implementing a simple RHexLib

program which periodically displays the encoder counters.

\section PrintModuleSection The PrintModule Class

In RHexLib, any task that needs to be performed periodically is defined as

a Module, more specifically as a class derived from the Module

abstract base class. Below is a definition of the  PrintModule class,

which encapsulates the task of periodically reading and printing the encoder

counter values.

\code

class PrintModule : public Module {

public:

PrintModule ( void ) : Module( "print", 0, false, false ) { };

void init( void ) { };

void uninit( void ) { };

void activate( void );

void deactivate( void );

void update( void );

};

\endcode

Note that each class derived from the virtual base class Module must

define five methods: init, uninit, activate, deactivate and

update. These methods provide a a standard interface to the \ref

ModuleManagerGroup "module manager" which, among other tasks, keeps a

list of all modules, manages their states, and calls the above methods

at appropriate times. In the case of PrintModule, the uninit method

does not do anything, the rest of the methods are defined below.

An important detail to note is that the  Module class constructor takes

four arguments. The first two determine the name and index of

the module, which are used together to uniquely identify each module in the

system. The last two arguments determine the type of the module and

are explained in later chapters of this manual.

\section InitMethodSection The init method

Module::init is called whenever the module is added to the module

manager's list with a call to the function MMAddModule(). The init

method is usually responsible for initializing related hardware

components and finding components and other modules that are needed

for the module's operation. In our example, no initializations are

required, so the init method is empty.

\section ActivateMethodSection The activate method

Module::activate is called whenever the module is activated by

the module manager via a call to either MMActivateModule() or the first

call to MMGrabModule(). This changes the the module from the INACTIVE

state to the ACTIVE state, which means that the module manager will

begin to periodically call the update method of the module.

Activating a module usually involves making sure that all the

components needed for the task are active and operational. Moreover,

necessary initializations of the components must also be

performed. For our example, the PrintModule will enable all the

encoders using the hardware interface.

\code

void PrintModule::activate( void ) {

int axis;

for ( axis = 0; axis < 6; axis++ )

EncoderHW::instance()->enable( axis );

}

\endcode

An important detail to note is the way hardware components are

accessed. Interfaces to hardware components are defined as abstract

base classes (such as EncoderHW, DCMotorHW, DriveHW etc.). Each of

these classes internally maintains a single instance of an actual

implementation, which is initialized by the particular hardware

library to be used. For those of you who are familiar with design

patterns, this corresponds to a singleton design pattern. To be able

to use a particular hardware component, the corresponding header file

("hardware/EncoderHW.hh" in this case) must be included.

As you may have guessed, the instance() method of such classes returns

a pointer to the singleton instance of the corresponding hardware

component, which can then be used to access the hardware

functionality. A more detailed treatment of how hardware access is

handled by RhexLib can be found in \ref HardwareGroup .

\section DeactivateMethodSection The deactivate method

Module::deactivate is called whenever the module is deactivated

through a call to either MMDeactivateModule() or the last call to

MMReleaseModule(). This puts the module in the INACTIVE state, which

means that the update method of the module will not be called anymore.

For our example, upon deactivation, the  PrintModule will disable all

the encoders using the hardware interface.

\code

void PrintModule::deactivate( void ) {

int axis;

for ( axis = 0; axis < 6; axis++ )

EncoderHW::instance()->disable( axis );

}

\endcode

\section UpdateMethodSection The update method

Module::update is where the core functionality of a module is

defined. When a module is in the ACTIVE state, its update function is

called periodically by the module manager.

Our example module, the  PrintModule, will read the encoders and print

their contents in its  update method. Moreover, it will also be

responsible for detecting keyboard input and exiting the program.

\code

void  PrintModule::update ( void ) {

int axis;

printf( "Encoders: " );

for ( axis = 0; axis < 6; axis++ )

printf( "0x%04x, ", EncoderHW::instance()->read( axis ) );

printf( "\n" );

if ( kbhit() ) {

MMMessage( "User interrupt: Shutting down!\n" );

MMPowerOff( );

}

}

\endcode

\section UsingPrintSection Using PrintModule in a program

What makes modules operational in RHexLib is the

\ref ModuleManagerGroup "Module Manager". In order to be operational,

modules must be created as instatiations of the class, and the module

manager must be informed of its presence through the

MMAddModule() library call. Moreover, it needs to be activated through

the  MMActivateModule() or  MMGrabModule() library functions in

order for its update function to start being called periodically.

The library function MMMainLoop() is the main entry point to the module

manager, and only returns when the program is about to terminate. It

is usually called once from main(), after all the desired modules are

created and added to the module manager's list. Once this function is

called, the module manager takes control and handles the periodic

issues of module updates as well as other timing tasks.  Following a

call to MMMainLoop(), the only way to exit the program is through the

MMPowerOff function, from within one of the module methods.

As such, there is a very typical way in which RHexLib programs are

built. The following program is the main entry point to our small

example and demonstrates how things are setup in a typical RHexLib

program..

\code

#include "ModuleManager.hh"

int main( void ) {

PrintModule pr; // Create an instance of the PrintModule module.

// Initialize whichever hardware library we are linked against

initHardware();

// Add and activate the print module

MMAddModule( &pr, 10, 0, 10 );

MMActivateModule( &pr );

// Call the Module Manager main loop

MMMainLoop();

// Cleanup underlying hardware before we exit

cleanupHardware();

// Let the module manager do its final cleanup

MMShutdown();

return 0;

}

\endcode

As this code segment shows, the typical structure of  main() is as

follows:

- call initHardware() to initialize underlying hardware interfaces

- Create and initialize all modules that will be used by the program.

- Add the newly created modules to the module manager's list through

MMAddModule() function calls.

- Activate the necessary modules through  MMActivateModule()

function calls

- Call MMMainLoop()

- Call cleanupHardware() to cleanup underlying hardware interfaces

- Call MMShutdown() to let module manager do its final cleanup

During the call to MMMainLoop(), the module methods will determine the

flow of the execution. Note that at least one module needs to be

activated before MMMainLoop is called. Otherwise, the program will be

stuck in an infinite loop, and no module updates will take place.

An important detail is that the parameters of a module related to the

scheduling of its updates are determined by the arguments to the

MMAddModule() function call. It is hence possible to configure the

period, the starting time offset and the update order (in case of

multiple updates scheduled at the same time) of each module.

*/