/** \defgroup DatabaseGroup Database (Blackboard) Facilities

One of the major applications of the communications manager is to

publish robot information and parameterize robot operations.  To

facilitate this, we have built a database abstraction on top of the

communication facilities that allows a user to create a central

database of named entries, where remote clients can subscribe to the

entries or register to change the entries by name, without having to

know anything about the details of the communications manager.

The database is owned by a central manager, typically residing on the

robot.  Database clients, such as graphical user interfaces, connect

to the database manager and register to either subscribe to database

entries (i.e., when the entry values change on the robot, the new

values are propagated to the clients) or to publish to database

entries (i.e., when the entry value changes on the client, the new

values are propagated to the manager).  Clients cannot register to

both publish and subscribe to the same entry.

There are two methods by which information can be propagated between

database clients and managers.  In immediate mode, as soon as a change

is made, that change is propagated.  Changes made in immediate mode

can be throttled to save network bandwidth, for example, a limit can

be set to send changes in a database entries at most every n

milliseconds.  In periodic mode, the value of the database entry is

propagated over the network at a regular interval.

Changes in database entries are propagated by both database managers

and clients through a subthread which contains a list of database

entry monitors.  When a database entry is marked as having an external

``client'', a monitor is added to the list.  The subthread iterates

through the list at a regular interval to check all of the monitors.

Periodic monitors fire at regular intervals and send the current value

to the recipient.  Immediate monitors simply check to see if there is

a throttled change pending to send.

Users can also add callbacks to entries which invoke a C++ method when

a database entry is changed.  These callbacks are implemented in the

simplest possible manner, which means they are very inefficient, and

really should not be used.  When you add a callback to an entry, a

monitor gets added to the monitor list.  Whenever the monitor list

checks this monitor, it will perform a comparison of the old entry

value and the new entry value.  If they are different, then the

callback is marked to be executed by a update function that the user

of both data base managers and clients must call.  Note that this

means that callbacks will not be invoked immediately, add significant

computational burden to the monitor thread, and require the user to

periodically invoke a special method in the ``main'' thread of

control.  It is my opinion that callbacks should not be used at all,

and that you should design your code accordingly.  If there is a great

hue and cry for them, these drawbacks can be alleviated somewhat, but

it will take more restructuring of code than you think to make them

efficient and effective.

Although the individual components have their classes defined in a

variety of header files, you will only have to include the file

Database.hh.

<B>Database Entries</B>

The core of the database are the named database entries, encapsulated

within the class \ref DBEntry.  First, each entry has a name by which

it is known in the database.  What the entry contains is essentially a

block of bytes: You can consider a database entry as ``wrapping'' an

instance of a communiations manager message, with all of the caveats

about byte order (must be identical across processes), structure

alignment (must be compatible between compilers), and structure

components (there can be no pointers unless you pack the data

yourself).

In addition, each entry has a ``description.''  The purpose of a

description is to give some measure of run-time type checking for the

data that goes into and out of the entry as remote clients subscribe

to it.  Thus, the description is usually the name of the type which

resides in the entry data.  For example, if a database entry simply

contains an integer, its description would be ``int.''  The entry

converts the description into a single integer key for comparison.

Note that database entries can be even more expensive than

communication mailboxes and streams, and thus it is imperative that

the database not be simply a bunch of named integers and doubles.

Data that is meant to be used together should be grouped together into

structures as much as possible.

<B>Common database operations</B>

Both the database manager and the database client classes have a

common superclass, \ref DBBase, which contains methods common to both,

such as the creation and management of the monitor thread or the

management of callbacks.

<B>Database Manager</B>

There is one database manager (an instance of the class DBManager)

which ``owns'' all of the database entries.  Clients can access and

even add entries remotely , but the entries are still truly maintained

by the database manager.

As well as creating a monitor thread (through it's superclass,

\ref DBBase), a database manager also creates a protocol thread, which is

watching a stream sink for messages from database clients about access

to the manager's database.

- Database manager example (Needs to be done: Take a look at RHexLib/base/db_test.cc)

<B>Database Clients</B>

Users access the central database managed by a database manager by

using the database clients (through the \ref DBClient class).  These

clients send messages to the database manager's stream sink, and

performs all of the necessary handshaking to setup database

communications.  Unlike the database manager operations, which return

DBEntry's, the database client operations return a subclass,

RemoteDBEntry which allows the user to detect when the remote database

manager has gone down and come back up, and allows the user to quickly

reregister the entry with the new database manager.

Note there is one serious lack in the database interface: currently

while remote clients can register to subscribe to database entries,

there is no way for them to cancel a subscription.  The database

manager is smart enough to see that an client has stopped and

restarted, and will ``cancel'' all of the client's previous

subscriptions (thus the necessity of the RemoteDBEntry class), so this

is not a fatal flaw, but it is still a flaw.

% need to get focused example here

- Database client example (Needs to be done: Take a look at RHexLib/base/db_test.cc)

*/