/** \defgroup HardwareGroup Hardware abstraction layer

A robot consists of a large variety of hardware: motors, encoders, analog and

digital input and output, etc.  RHexLib provides a uniform mechanism for

defining and accessing these pieces of hardware in an abstract (to ease porting

to different devices or from hardware to simulation) but uniform (to promote

code maintainability and reduce bugs) manner.

Specifically, each type of hardware can be defined as a class which has

operations specific to that type of hardware.  RHexLib provides a \ref Hardware

template to implement the uniform interface mechanisms.  Developers then use

the HARDWARE_DECL() macro to declare their hardware class, and use the

HARDWARE_IMPL() macro when implementing it, and the resulting class has all of

the \ref Hardware template's functionality in addition to the hardware specific

functionality.

Then, in a hardware specific library, the developer must create the functions

initHardware() and cleanupHardware().  initHardware() is where specific

concrete instances of hardware are allocated and created and registered as

"hardware instances" using the Hardware::registerInstance static methods.

After initHardware() is called, developers can simply use the static

Hardware::instance() method to access that instance.

cleanupHardware() is called at the end of the program to deallocate and destroy

all of the hardware instances created in initHardware().

In addition, the Hardware template provides rudimentary facilities for

"grabbing" and "releasing" hardware instances.  This may be useful in order to

mark a piece of hardware which, for physical reasons, should only have one user

at a time, such as a digital output.  We also provide for "multi-channel"

hardware instances, such as a multi-channel A/D board.  Individual channels can

then be grabbed and released by passing in a channel index.

When running with different physical hardware, the developer simply replaces

the hardware library with another that creates different concrete hardware

instances, but which registers them in a manner consistent with the "old"

physiucal hardware.  To the user of the hardware classes, there should be

no need for change for a different set of "real" hardware.  In addition, ths

approach allows incremental addition of novel hardware, as opposed to a single

hardware abstraction which, hopefully, covers all possible types of hardware.

*/