/** \defgroup ModuleManagerGroup Module manager facilities

RHexLib provides simple means for defining tasks that need to be

performed periodically. Following initialization, all RHexLib programs

surrender control to the "module manager", which performs synchronous

scheduling of periodic tasks defined through instantiations of the

\ref Module class. All module manager functionality is accessed

through a collection of functions (all prefixed with "MM"), described

in this section.

The \ref Module abstract base class defines a template for periodic

tasks that can be defined by a RHexLib programmer. Defining such a

task involves deriving a class from \ref Module, and providing

implementations for its methods \ref Module::init, \ref

Module::uninit, \ref Module::activate, \ref Module::deactivate and

\ref Module::update. In particular, the update method is periodically

called by the module manager and defines the core functionality of the

module.

The module manager maintains a database of currently recognized

modules. Each module in the database is uniquely identified with its

name and index, specified through its constructor \ref

Module::Module. Usually, the functions \ref MMAddModule and \ref

MMRemoveModule are called from within main() to initialize this

database. The \ref MMFindModule function can be used to query this

database.

The execution properties of a module are parametrized with three

quantities: <em>period</em>, <em>offset</em> and <em>order</em>. As

noted before, the module manager is a "synchronous" scheduler. As

such, it divides time into "steps" of equal duration (usually 1ms),

and schedules module updates within these slots. Both the period and

offset of a module are integers corresponding to these steps and

respectively specify the periodicity of module updates and the

beginning time offset of the first update (for periods larger than

1). Finally if multiple modules are to be updated in the same step,

the order parameter determines the execution order with lower values

having higher priority. These parameters can be specified through the

\ref MMAddModule function call.

Modules can be activated and deactivated through the functions \ref

MMActivateModule and \ref MMDeactivateModule. However, it is much

better practice to use the functions \ref MMGrabModule and \ref

MMReleaseModule instead as they provide access control and reference

counting for activation and deactivation. These two functions provide

reliable means of resource management for modules.

The function \ref MMMainLoop must be called from main() to transfer

execution control to the module manager. Beyond that point, module

scheduling starts and the module manager retains execution control

until either \ref MMShutdown or \ref MMPowerOff are explicitly called

from within module methods.

The module manager also provides message facilities either through the

console display or through a network connection to a message logger

client. The functions \ref MMMessage, \ref MMWarning and \ref

MMFatalError can be used to issue messages at various levels of

priority and severity.

The following configuration file symbols are recognized by the module manager:

\li \b int mm_update_period: Time in between two module updates, given in microseconds. This determines the fundamental frequency of module updates.

\li \b float mm_mainloop_prio: QNX process execution priority for

the main module manager loop.

\li \b float mm_min_idle_time: Minimum idle time per step to be

left unused by the main module manager loop process (seconds). This

feature ensures that enough time is left for other systems processes

such as communications and serial device access.

\li \b float mm_profile_enable: When nonzero, enables execution

profiling for module updates.

\li \b string mm_profile_filename: When module execution

profiling is enabled, this variable specifies the output filename for

execution statistics.

\li \b float mm_msglog_enable: When nonzero, enables logging of

module manager messages to a local file on the hard disk.

\li \b string mm_log_filename: When local message logging is

enabled, thois variable spacifies the message dump filename.

\li \b float mm_log_priority: When local message logging is

enabled, this variable specifies the priority threshold for message

filtering.

*/