/** \page SupMachinePage State machines: A supervisor controller

In this section we construct a more complicated state machine, called

Supervisor and shown in the figure below. The basic job of the

Supervisor machine is to activate and deactivate lower level

controllers, which may be state machines themselves, based on certain

events. This particular Supervisor machine pays attention to remote

control events reported by the RemoteControl module. The controllers

it activates make the machine calibrate, stand, sit and do

pushups. The first three behaviors are governed by the standard

RHexLib modules CalibMachine, StandMachine and SitMachine. The last

behavior is governed by the PushupController constructed in a previous

tutorial.

Note that this custom implementation of the supervisor is different

from the facilities provided by the ModeSupervisor, which imposes a

very particular structure on the transitions between different

behaviors and the nature of user commands through a network

channel. Finally, as many of the components used by this supervisor

are RHex specific, we are not able to provide a complete

implementation in the examples directory. Most of the modules that are

used in the implementation are not part of the core distribution.

\image html sup.gif  A state machine, called  Supervisor, which is implemented in this section. This machine ``supervises'' the activity of other controllers, including other state machines. The events are either command events (which check the remote control state) or time events. Based on the remote control activity, the machine calibrates, stands, sits or does ``pushups''.

\image latex sup.eps  A state machine, called  Supervisor, which is implemented in this section. This machine ``supervises'' the activity of other controllers, including other state machines. The events are either command events (which check the remote control state) or time events. Based on the remote control activity, the machine calibrates, stands, sits or does ``pushups''.

The transition table that corresponds to Figure above, and which

should be saved in a file called Supervisor.dsc is as follows:

\code

Transition uncalibrated calCommand calibrating

Transition calibrating calFail uncalibrated

Transition calibrating calSuccess idle

Transition idle standCommand runningStand

Transition runningStand standDone standing

Transition standing sitCommand runningSit

Transition runningSit sitDone idle

Transition standing pushupCommand doingPushups

Transition doingPushups stopCommand runningStand

Initial uncalibrated

\endcode

There are seven states.  uncalibrated is the initial state,

corresponding to the situation that the robot has just been turned on.

calibrating is the state the robot will be in while running a

calibration state machine for each leg. In the idle state, the robot

will be sitting with its motor drives deactivated to save power. In

the runningStand state, the robot will be running the standMachine

state machine which brings the robot to a standing position. In

standing state, the robot will have completed the standMachine and

will be waiting to either sit or start doing pushups. In the state

doingPushups, the robot will be running the pushupController from the

previous tutorials. Finally, in runningSit, the robot will be running

the SitMachine controller.

There are five ``command'' events which will correspond to remote

control events. First, the calCommand::check method, which signals the

user's intention to calibrate the robot's legs, will be true when the

left joystick is pushed forward. The standCommand::check method, which

signals the user's intention to stand the robot up from idle mode,

will be true when the left joystick is pushed backward. The

pushupCommand::check method, which signals the user's intention to

start the pushup controller from standing mode, will be true when the

left joystick is pushed forward. The stopCommand::check method, which

signals the user's intention to stop the pushup controller, will be

true when the left joystick is pushed backward. The sitCommand::check

method, which signals the user's intention to make the robot sit when

in standing mode, will be true when the left joystick is pushed to the

left.

The other events, calibSuccess, calibFail, standDone and sitDone,

correspond to the various controllers coming to completion. For

example, when the StandMachine has finished, its isDone flag returns

true. Thus, standDone::check calls this method. As the figure shows,

while each of the controllers is running, the Supervisor will be in a

mode that is waiting for a signal that the controller has completed,

before moving on to a state that waits for the next user input. The

exception is the doingPushups state, which is terminated upon a remote

control action from the user.

\section codingsec Coding the Supervisor

We will see that there are very few things to be done to code up this

seemingly complex state machine. First, we use sm-setup.pl to create

skeleton header and code files from the Supervisor.dsc file. This

creates 90\% of the code for the Supervisor machine.

The header, Supervisor.hh, is practially finished. We just need to

include certain header files from RHexLib which define the other

machines to be used and add variables to the Supervisor class to point

to the modules it will need. We also suppose that PushupController.hh

has been created and placed in the same directory as the supervisor

code. The resulting header file is as follows:

\code

#ifndef _SUPERVISOR_HH

#define _SUPERVISOR_HH

#include "ModuleManager.hh"

#include "StateMachine.hh"

// The next six includes have been added to the basic

// file produced by sm-setup.pl

#include "StdModules.hh"

#include "StandMachine.hh"

#include "SitMachine.hh"

#include "CalibMachine.hh"

#include "RemoteControl.hh"

#include "PushupController.hh"

class Supervisor : public StateMachine {

public:

Supervisor ( void );

~Supervisor ( void );

void init ( void );

void activate ( void );

void deactivate ( void );

private:

// events

EventObject ( CalCommand ) * calCommand;

EventObject ( CalFail ) * calFail;

EventObject ( CalSuccess ) * calSuccess;

EventObject ( StandCommand ) * standCommand;

EventObject ( StandDone ) * standDone;

EventObject ( SitCommand ) * sitCommand;

EventObject ( SitDone ) * sitDone;

EventObject ( PushupCommand ) * pushupCommand;

EventObject ( StopCommand ) * stopCommand;

// states

StateObject ( Uncalibrated ) * uncalibrated;

StateObject ( Calibrating ) * calibrating;

StateObject ( Idle ) * idle;

StateObject ( RunningStand ) * runningStand;

StateObject ( Standing ) * standing;

StateObject ( RunningSit ) * runningSit;

StateObject ( DoingPushups ) * doingPushups;

// These six pointers have been added to the basic file produced

// by sm-setup.pl. They will point to other modules used by the Supervisor

PushupController * puControl;

StandMachine     * standMach;

SitMachine       * sitMach;

CalibMachine     * cm[6];

RemoteControl    * rc;

};

#endif

\endcode

The next step is top modify the code file,  Supervisor.cc, produced by

sm-setup.pl. We will do this in several steps:

\li Modify Supervisor::activate() to search for the required

modules (for standing, sitting, etc.) and set up the module

pointers that we added to Supervisor.hh.

\li Modify Supervisor::activate() and Supervisor::deactivate() to

grab and release the remote control module.

\li Modify the event check methods to check for remote control

events and controller isDone events.

\li Modify the state entry, during and exit methods (not all of

them) so that they activate and deactivate the various modules

responsible for calibrating, standing, sitting and doing pushups.

First we modify the init method to search for the required modules:

\code

void Supervisor::init ( void ) {

int i;

// this is where to search for the needed modules and set up pointers to them

if ( ( standMach = ( StandMachine * )

MMFindModule( STANDMACHINE_NAME, 0 )) == NULL)

MMFatalError ( "Supervisor::init", "Cannot find Stand Machine" );

if ( ( sitMach = ( SitMachine * )

MMFindModule( SITMACHINE_NAME, 0 )) == NULL)

MMFatalError ( "Supervisor::init", "Cannot find Sit Machine" );

if ( ( rc = ( RemoteControl * )

MMFindModule( REMOTECONTROL_NAME, 0 )) == NULL)

MMFatalError ( "Supervisor::init", "Cannot find Remote Control" );

if ( ( puControl = ( PushupController * )

MMFindModule( "pushupcontroller", 0 )) == NULL)

MMFatalError ( "Supervisor::init", "Cannot find Push Up Controller" );

for ( i = 0; i < 6; i++ )

if ( ( cm[ i ] = ( CalibMachine * )

MMFindModule( CALIBMACHINE_NAME, i )) == NULL)

MMFatalError ( "Supervisor::init", "Cannot find Calibration Machine" );

StateMachine::init();

}

\endcode

Next, we modify the activate and deactivate methods so that they

grab, configure and release the remote control module. The arguments

to rc->configure are the delays of the right and left joystick

inputs. A deley of 0.005$s$, for example, means that the value has to

be stable for 0.005 seconds before it is reported. The

rc->setThreshold method sets the value above which the joystick is

considered to be pushed in a particular direction. In the deactivate

method, we also check to see if any of the modules that the Supervisor

uses are active, and if they are, we deactivate them.

\code

void Supervisor::activate ( void ) {

// Configure the RC sticks

rc->configure ( 0.005, 0.3 );

rc->setThreshold ( 0.5 );

// Grab and activate the remote control interface

MMGrabModule( rc, this );

}

void Supervisor::deactivate ( void ) {

// Release the previously grabbed modules

MMReleaseModule( rc, this );

if ( standMach->getState() == MODULE_ACTIVE ) MMReleaseModule ( standMach, this );

if ( sitMach->getState() == MODULE_ACTIVE ) MMReleaseModule ( sitMach, this );

if ( puControl->getState() == MODULE_ACTIVE ) MMReleaseModule ( puControl, this );

for ( i=0; i<6; i++ )

if ( cm[i]->getState() == MODULE_ACTIVE ) MMReleaseModule ( cm[i], this );

StateMachine::deactivate();

}

\endcode

\section eventchecksec The Event check Methods

Next, we add meaning to the event check methods. We start with

calibCommand. This amounts to checking the value returned by the

rc->leftSick and rc->rightStick methods. The values returned by these

methods correspond to the cardinal map directions (north, west,

northwest, etc.)  Thus, the calibCommand::check method below returns

true if the left joystick is pushed directly forward.

\code

bool Supervisor::CalCommand::check ( void ) {

// Left RC stick pushed forward

return bool ( OWNER->rc->leftStick() == RemoteControl::NORTH );

}

\endcode

The next two events check for either the sucessful completion of all the

calibration machines (one for each leg) or the unsucessful completion of at

least one of them.  CalibMachine::getStatus returns

CalibMachine::FAILURE in the case that calibration failed,

CalibMachine::CALIBRATING if calibration is still underway and

CalibMachine::SUCCESS if calibration suceeded.

\code

bool Supervisor::CalFail::check ( void ) {

int i;

// return true if any motor reports failure

for ( i = 0; i < 6; i++ )

if ( OWNER->cm[i]->getStatus() == CalibMachine::CALIBRATING ) return false;

for ( i = 0; i < 6; i++ )

if ( OWNER->cm[i]->getStatus() == CalibMachine::FAILURE ) return true;

return false;

}

bool Supervisor::CalSuccess::check ( void ) {

// return true if all motors report success

int i;

for ( i = 0; i < 6; i++ )

if ( OWNER->cm[i]->getStatus() != CalibMachine::SUCCESS ) return false;

return true;

}

\endcode

The stand command is issued by pushing the left joystick directly

backward.

\code

bool Supervisor::StandCommand::check ( void ) {

// Left RC stick pushed back

return bool ( OWNER->rc->leftStick() == RemoteControl::SOUTH );

}

\endcode

Via the public interface to  StandMachine, other modules can

perceive two states. Either the machine is in the midst of standing, in which

case  StandMachine::isDone returns  false or the machine is

completed its task and is holding steady, and  isDone returns  true.

\code

bool Supervisor::StandDone::check ( void ) {

return OWNER->standMach->isDone();

}

\endcode

To sit, push the left joystick directly to the left.

\code

bool Supervisor::SitCommand::check ( void ) {

// Left RC stick pushed left

return bool ( OWNER->rc->leftStick() == RemoteControl::WEST );

}

\endcode

The  SitMachine interface is similar to the  StandMachine interface.

\code

bool Supervisor::SitDone::check ( void ) {

return OWNER->sitMach->isDone();

}

\endcode

To start doing pushups, push the joystick directly forward while in

standing mode.

\code

bool Supervisor::PushupCommand::check ( void ) {

// Left RC stick pushed forward

return bool ( OWNER->rc->leftStick() == RemoteControl::NORTH );

}

\endcode

To stop doing pushups, push the left joystick directly backward.

\code

bool Supervisor::StopCommand::check ( void ) {

// Left RC stick pushed back

return bool ( OWNER->rc->leftStick() == RemoteControl::SOUTH );

}

\endcode

\section statemethsec The State Methods

The next and final step is to modify the state methods. Note that we do not

modify them all as they are not all needed. For example, none of the

during methods are changed at all as will often be the case in supervisor

state machines. To see how the  during methods are used, see the code of,

for example,  StandMachine. Here, only  some of the  entry and

exit methods are actually required. Mostly what they do is activate and

deactivate other modules. Activation of a module is done with the

MMGrabModule function. Deactivate is done with the  MMReleaseModule

function.

There are six caibration machines to be activated upon entry into the

calibrating state as follows. This method also sets the calibration method of

each machine to  GROUND which makes the robot's legs spin until the meet

with some resistance (persumably the ground).

\code

void Supervisor::Calibrating::entry ( void ) {

int i;

for ( i = 0; i < 6; i++ ) {

OWNER->cm[i]->setMode( CalibMachine::GROUND );

MMGrabModule ( OWNER->cm[i], owner );

}

}

\endcode

Upon completion of the calibrating state, when either calibSucess or

calibFail become active, we release the calibration machines, thereby

deactivating them.

\code

void Supervisor::Calibrating::exit ( void ) {

int i;

for ( i = 0; i < 6; i++ )

MMReleaseModule ( OWNER->cm[i], owner );

}

\endcode

Successful calibration leaves the robot calibrated with the motor

drives off. Thus, to start standing, we need to enable the motor

drives and then grab the stand machine.

\code

void Supervisor::RunningStand::entry ( void ) {

int i;

for ( i = 0; i < 6; i++ )  // enable motor drives

DriveHW::instance()->driveEnable( i, true );

MMGrabModule ( OWNER->standMach, owner );

}

\endcode

We leave the standing machine running during the standing state

though it is in the standing position. This is because the

StandMachine holds the robot in the upright position after it is

done. We release (and deactivate) the standing machine only when

leaving the standing state on our way either to sitting or doing

pushups.

\code

void Supervisor::Standing::exit ( void ) {

MMReleaseModule ( OWNER->standMach, owner );

}

\endcode

To start sitting, we grab the sit machine (don't let anyone just anyone grab

your sit machine!).

\code

void Supervisor::RunningSit::entry ( void ) {

MMGrabModule ( OWNER->sitMach, owner );

}

\endcode

When the  SitMachine is done, we release it and also disable

the motor drives to save power.

\code

void Supervisor::RunningSit::exit ( void ) {

int i;

for ( i = 0; i < 6; i++ )  // disable motor drives

DriveHW::instance()->driveEnable( i, false );

MMReleaseModule ( OWNER->sitMach, owner );

}

\endcode

Finally, it is easy to incoorporate the pushup controller, we just grab and

release it as needed:

\code

void Supervisor::DoingPushups::entry ( void ) {

MMGrabModule ( OWNER->puControl, owner );

}

void Supervisor::DoingPushups::exit ( void ) {

MMReleaseModule ( OWNER->puControl, owner );

}

\endcode

\section minfilesec A main() File

One more piece is required to get the Supervisor runnning. We need a

main file. The following will do.

\code

#include <stdio.h>

#include "sysutil.hh"

#include "ModuleManager.hh"

#include "StdModules.hh"

#include "StateMachine.hh"

#include "Supervisor.hh"

#include "RemoteControl.hh"

#include "PushupController.hh"

class UserModule : public Module {

public:

UserModule ( void ) : Module( "userinput", 0, false, false ) { };

void  init ( void ) {}

void  uninit ( void ) {}

void  activate ( void ) {}

void  deactivate ( void ) {}

void  update ( void ) {

int i;

if ( kbhit() ) {

// Diable all motor drives

for ( i = 0 ; i < 6 ; i++)

DriveHW::instance()->driveEnable ( i, false );

MMShutdown();

MMPowerOff( );

}

}

} umod;

int main ( void ) {

initHardware();

// Read robot specific configuration file

MMReadConfigFile( "rhex_michigan.rc" );

// Read the application dependent configuration file

MMReadConfigFile( "sup.rc" );

Supervisor supervisor;

PushupController puc;

// Create and add all the standard modules

RHexAddStdModules();

// Add the user module, pushup controller and supervisor

MMAddModule ( &umod, 1, 0, USER_CONTROLERS );

MMAddModule ( &puc, 1, 0, USER_CONTROLLERS );

MMAddModule ( &supervisor, 1, 0, USER_CONTROLLERS );

// Activate the supervisor modules

MMActivateModule ( &supervisor );

MMMainLoop();

cleanupHardware();

MMShutdown();

return 0;

}

\endcode

The machines for calibration, standing and sitting as well as the

remote control module are all added to the Module Manager by the command

RHexAddStdModules. Only the  Supervisor and the  PushupController

are added in  main(). Also, only the  Supervisor module is activated

in  main(). It takes care of activating everything else it needs. We have

also added a simple module, called  UserModule, which allows the user to

stop the program by pressing any key.

Note, we have chosen to configure this program to use the

rhex_michigan.rc file.

*/