/** \page CDLTutorial  Creature Description Language Tutorial

This page serves as the basis for learning how to encode robot designs as CDL files, for use with the RHexLib simulation environment.

\li \ref CDLTutorialExample

\li \ref CDLTutorialSyntax

\li \ref CDLTutorialNumerical

\li \ref CDLTutorialFrames

\li \ref CDLTutorialGeom

\li \ref CDLTutorialJoints

\li \ref CDLTutorialGraphics "Renderer Options"

\li \ref CDLTutorialOther "Other Functions"

\li \ref CDLTutorialAdvanced "Advanced Examples"

\li \ref CDLTutorialInterfacing "Using the CDL"

\li \ref CDLTutorialPreproc "Preprocessors for Macro Expansion"

\li \ref CDLTutorialRevisions "Documentation Revisions"

*/

/** \page CDLTutorialExample  Introductory Example

Before we even start learning the language, here is an example of the CDL in action.  The following describes a weighted block, able to fall freely once the simulation starts.

\code

Camera(5,5,5);

LookAt(0,0,0);

Gravity(0,0,-10);

DefineSurface("default");

FreeBody, Block(1,1,1) : (mass=1.0, color="red");

\endcode

The first four lines set the simulation up, defining the camera, gravity, and the default surface.  The last line of the code is what sets the simulation in action.  A single block, of size 1 meter on all sides, is suspended in the air and set free.  The Block has a mass, and a color is set for its surface.

\image html tutorial01.jpg "falling block, 0 sec"

\image html tutorial02.jpg "falling block, 2/3 sec"

\image html tutorial03.jpg "falling block, 4/3 sec"

*/

/** \page CDLTutorialSyntax CDL Syntax and Semantics

The delimiters of the CDL are based upon C, so the language will look familiar upon first sight.  However, these characters often have much different meanings within the CDL, and there are additional delimiters used within the CDL.

As the syntax of a language is often one of the most complicated parts, this document has been split up into the following sections:

\li \ref SecBasics

\li \ref SecBraces

\li \ref SecCommas

\li \ref SecMixing

\section SecBasics Operations, Arguments, and Parameters

The most basic entity is an operation.  The following are all operations.

\code

FreeBody;         /* operation without arguments */

Block(1,1,1);     /* operations with arguments */

block(1,1,1);

Block(1,1,1) : (color="red");  // operations with parameters

Block(1,1,1) : (mass=1.0, surface="sticky");

\endcode

The first line is an operation without any arguments.  Most operations will require some arguments (just like function calls in C), but some do not require them.  In this case, you do not need to add parentheses after the operation's name.

The second line is a plain operation, including arguments.  The arguments tell the block it's size.  The CDL is case-insensitive, so the example given on the third line is semantically identical to the example on the second line.

While arguments hold the required, nameless values passed to an operation, a different construct, that of parameters, is used to pass additional values.  On the fourth line, delimited by a colon and surrounded by parentheses, is passed a single parameter, telling the block what color it should be.  Note that unlike arguments, parameters must be named.  The last line shows a similar operation, but with multiple parameters.

As seen in the example, comments can be added in either C or C++ notation.

\section SecBraces Braces to Change Reference

\code

Block(1,1,1) {

Translation(0,0,5) {

Block(2,2,2);

}

}

\endcode

Unlike in C, where braces serve as grouping operators, for instance to form functions (which are currently non-existent in the CDL as of this writing), braces in the CDL change the frame of reference for future operations.

In this example, the translation operation is performed within the reference of the first block operation.  The second block is performed within the reference of the translation.  You can read these operations linearly, so you apply the first block, then you apply the translation, and finally, you apply the last block.

The world always starts at (0,0,0).  Subsequent operations can add objects to the world, apply rotations and translations, or define interconnections between objects.  In the example above, the first block is placed at (0,0,0), while the second block is placed at (0,0,5), after the translation has moved the reference frame.

\section SecCommas Commas as Alternative Notation

The following is semantically identical to the above code:

\code

Block(1,1,1), Translation(0,0,5), Block(2,2,2);

\endcode

This comma notation grew out of the need for a shorthand version to write many operations called linearly.  Quite often when defining geometries, objects are organized as chains of operations, meaning that you would have a host of braces to close once you got to the end of your list of operations.  After the semicolon at the end of the line, you are returned to the original reference frame, before the first block.

\section SecMixing Mixing Braces with Commas

It is possible to mix notations, but you must always be careful as to what reference frame you are at.  As seen in the previous section, a semicolon at the end of a comma-separated list returns you to the reference frame at the beginning of the list.  How about with braces?  The specialty of braces is that they allow you to place multiple operations at the same frame of reference.  As long as you are within a set of braces, you will always be at that frame of reference, at least.  Consider the following example (using fictitious operations for clarity).

\code

A {

B {

C,D,E;

F;

}

G,H;

}

\endcode

\b A starts us out at the original reference frame.  \b B is then performed within \b A's reference frame, and we are pushed to its reference frame.  \b C, \b D, and \b E are then all performed linearly, starting from \b B's reference frame.  The semicolon then pops us back to \b B's reference frame, where \b F is performed.  You can think of \b F and \b C as being performed in parallel, from the same reference frame.

The first brace closes, popping us back to \b A's reference frame, in which \b G and \b H are applied, in linear order.  Finally, we are popped back to the original reference frame when the last brace closes.

I say push and pop because that is exactly what the language is doing.  It is constantly modifying a stack of reference frames.  The push/pop paradigm is a common one in computer graphics, and is used in systems such as OpenGL and Postscript.

*/

/** \page CDLTutorialNumerical Numerical Calculations

In addition to allowing you to apply operations at different frames of reference, the CDL has the ability to evaluate numerical expressions.  As most people are very familiar with it, the syntax was modeled after Matlab, with one principal exception.  While Matlab works with complex numbers as default, the CDL is only capable of working with real numbers.  Any operations returning a complex number will give NaN instead of the desired number.  This is a known limitation, and is being considered for future improvements.

Numbers can be input as integers (e.g. 5), in floating point notation (3.1415926), or in scientific notation (3e8, 1.6726e-27).  They are all stored internally as double precision floating point numbers.  If you would like to store a number as a variable, you must first initialize the variable by preceding it with the keyword "var".

\code

var y;       // y is initialized without a value (defaults to 0)

var x = 1.0; // x is initialized with a value of 1.0

y = x + 1;   // y now has a value of 2

\endcode

It is a syntax error to initialize a variable twice.

Now that we know how to input and reuse numbers and variables, we can use them in numerical function calls and expressions.  The basic algebraic operators are all defined.  Of particular note is that exponentials are done with carats, a la Matlab, and not like C, which requires a separate function call.  The following are all valid expressions:

\code

var x = sqrt(9);

var golden = ( 1 + sqrt(5) ) / 2.0;

var mymin = min(x,golden);

var iszero = sin(pi/3) - sqrt(3)/2;

\endcode

"pi" is a predefined variable, and all trigonometric functions operate in radians.  A complete list of implemented functions, with descriptions, follows:

Trigonometric Functions

\li \b sin(a) : the sine of an angle

\li \b cos(a) : the cosine of an angle

\li \b tan(a) : the tangent of an angle

\li \b asin(x) : the arcsine of a value, NaN if abs(x) > 1

\li \b acos(x) : the arccosine of a value, NaN if abs(x) > 1

\li \b atan2(x,y) : the four quadrant arctangent, returning values between -pi and pi

\li \b csc(a) : the cosecant of an angle

\li \b sec(a) : the secant of an angle

\li \b cot(a) : the cotangent of an angle

Rounding, Sign, and Comparison Functions

\li \b ceil(x) : the nearest integer rounded up

\li \b floor(x) : the nearest integer rounded down

\li \b round(x) : the nearest integer

\li \b sign(x) : the sign

\li \b abs(x) : the absolute value

\li \b max(x,y) : the larger of two numbers

\li \b min(x,y) : the smaller of two numbers

Exponential Functions

\li \b sqrt(x) : the square root, NaN if x < 0

\li \b log(x) : the natural logarithm, NaN if x <= 0

\li \b log10(x) : the base 10 logarithm, NaN if x <= 0

\li \b exp(x) : the exponential, e^x

Other Functions

\li \b rand(y) : a floating point random number between 0.0 and y (\b NB: this differs from the Matlab notation)

This limited set of numerical functions forms a wide basis to build geometries for robot designs.  However, this list is by no means complete, and for that reason, it is extremely simple to modify the source to include additional functions.

*/

/** \page CDLTutorialFrames Rotations and Translations

As mentioned previously, the CDL maintains a stack of reference frames, and constantly pushes and pops frames as you apply operations.  There are only two operations which change the reference frame explicitly, \b Rotations and \b Translations.

\section SecRots Rotations

Rotations can take either four or nine arguments.  If given nine arguments, it is assumed that you are inputting a full rotation matrix.  This matrix will be checked, and an error will occur if the determinant is not +1.  An example of a full rotation matrix follows.

\code

Rotation( cos(a), -sin(a), 0, // a rotation in the XY plane

sin(a),  cos(a), 0,

0,       0, 1);

\endcode

A much simpler notation is that of axis-angle, which is what results if you supply only four numbers.  The first three specify the axis of the rotation, and the last supplies the angle.  A positive angle results in a counter-clockwise rotation around the axis.  The following is an example of an axis-angle rotation.

\code

Rotation(0,0,1,a); // same rotation as above

\endcode

\section SecTrans Translations

Translations are of course a lot simpler, taking only three arguments, and simply translating our frame of reference by that amount.

\code

Translation(0,1,2); // translate by +1 in y, +2 in z

\endcode

\section SecCaveat Cave Canem

It is important to note that the operation name for both rotations and translations is the noun, note the verb.  The following would both be syntax errors.

\code

Rotate(1,0,0,pi); // No! Bad!

Translate(1,1,1); // Wrong!

\endcode

\section Joining Transforms

The tricky part of all this comes when you must think through a series of operations, and realize where you end up in world coordinates.  Let's walk through one example.  When sitting down to design a robot, it is often useful to have a pad of paper and a pen nearby to sketch out the rotations and translations.

\code

Translation(0,0,1) {

Rotation(0,0,1,pi/2) {

Translation(1,0,0) {

Rotation(1,0,0,pi) {

/* where are we now?? */

}

}

}

\endcode

At the reference frame of the last operation, the axis is centered at (0,1,1), with z (local) pointing in the direction of -z (world), x (local) pointing in +y (world), and y (local) pointing -x.

As you can imagine, for later utility, it is extremely beneficial to document your CDL code, stating exactly where your axes are pointing, after you have performed a group of transforms.

*/

/** \page CDLTutorialGeom Basic Shapes

The CDL has a short list of various shapes one can use to input a robot's geometry.  Let's first walk through each of the operations, with a description of their arguments.  Then we'll discuss the parameters which are common to all of them.

\section SecShapes Shapes

Let's start with the one shape we've already seen.

\subsection SubBlock Block(x,y,z)

When given three arguments, a block of said dimensions will be oriented and centered at the current reference frame.  Note that you must first translate if you wish the block to be positioned with a corner at (0,0,0) in the current reference frame.

If given a fourth argument, it is assumed to be a radius by which to extrude the block, smoothing the sharp edges.  The difference can be seen in the following images.

\image html tutorial04.jpg Block(1,1,1)

\image html tutorial05.jpg Block(1,1,1, 0.25)

\subsection SubSphere Sphere(rad)

A sphere has only one required argument, its radius.  The sphere is placed at the current reference frame.

\image html tutorial06.jpg Sphere(1.0)

\subsection SubCyl Cylinder(h,rad)

A cylinder needs two arguments, its height and its radius.  The cylinder is centered at (0,0,0), and its height is along the z axis.  An optional third argument can supply an additional extruding radius, similar to how the edges of a block can be smoothed.

\image html tutorial07.jpg Cylinder(1.0,1.0)

\subsection SubConvex Convex Hulls

Unlike the above operations, which are single function calls, a convex hull is different, since it can include any number of vertices.  A convex hull is defined by two different operations, ConvexHull and Vertex(x,y,z).  The former has an optional argument, the extruding radius for the whole hull, and the latter requires three arguments, a position.  Vertex operations must be placed with the frame of reference of a ConvexHull.  Not doing so is a syntax error.

\image html tutorial08.jpg ConvexHull without extruding radius

\code

ConvexHull {

Vertex(1,0,0);

Vertex(0,1,0);

Vertex(-1,0,0);

Vertex(0,-1,0);

Vertex(0,0,1);

}

\endcode

\image html tutorial09.jpg ConvexHull with extruding radius

\code

ConvexHull(0.25) {

Vertex(1,0,0);

Vertex(0,1,0);

Vertex(-1,0,0);

Vertex(0,-1,0);

Vertex(0,0,1);

}

\endcode

\section SecShapeParams Common Parameters

Now that we are working with shapes, we have a few additional parameters we can pass to the operations.

\li \b name="name" : the name of the object.  This is stored internally in the simulator, so you can later retrieve the object by name, or reference it from other points in a CDL file.

\li \b color="colorname" : as of this writing, limited to green, yellow, red, blue and black.  Near future plans call for adding the ability to specify any color, as well as to specify surface textures.

\li \b mass=m : gives the associated object a mass of m.  When working with blocks, spheres, and cylinders, the mass is distributed according to the shape of the object.  When working with convex hulls, the mass is a point mass centered at (0,0,0) in the current reference frame.  Take this in mind when designing your convex hulls!  (i.e. don't make them too big, and don't offset them too much from (0,0,0)).

\li \b surface="surfacename" : this will tell the simulator which surface type to use for the object.  Surface types are defined using the DefineSurface operation, described later.  No other parameters are needed here at the object.

*/

/** \page CDLTutorialJoints Joints and Degrees of Freedom

\li \ref SecRigid

\li \ref SecRevolute

\li \ref SecPrism

\li \ref SecJointParams

\li \ref SecFreeBody

Now that we have a good handle on how to move about with our reference frames, and how to place objects as geometries within the world, let's learn how to make our objects move.  But before things can move, we have to learn what makes them rigid.

\section SecRigid Rigidly Connecting Objects

If you only use the operations we've learned so far, translations, rotations, and our various geometry operations, you will construct a completely rigid object.  In the following example, the first block is rigidly attached to the world at (0,0,0), and the sphere is rigidly attached to the first at (0.5,0,0).

\image html tutorial10.jpg A rigid arm

\code

Block(1.0, 0.1, 0.1) : (color="red") {      // this block is long along X

Translation(0.5,0,-0.5) {                 // blocks intersect at (0.5,0,0)

Block(0.1, 0.1, 1.0) : (color="green"); // this block is long along Z

}

}

\endcode

\section SecRevolute Revolute Joints

Now let's make this example into a double pendulum.  For this we need to use the RevoluteJoint operator.  This operator has three optional arguments (as of this writing, the latter two are not yet implemented).  The first argument is the starting position of the joint, in radians, and is assumed to be 0 if not given.  The latter two are optional joint limits, and the joint is unlimited if they are not given.

Following the robotics standard, revolute joints rotate about the Z axis.  For this reason, you must often rotate in and rotate out if you want to place a revolute joint in an existing design.  Let's do this for the above arm, previously rigid.

\code

Translation(-0.5,0,0),      // translate to endpoint of bar

Rotation(1,0,0,-pi/2),     // rotate Z so it aligns with world Y

RevoluteJoint,             // place a joint

Rotation(1,0,0,pi/2),      // rotate back

Translation(0.5,0,0) {     // translate back

Block(1.0, 0.1, 0.1) : (color="red", mass=1) {

Translation(0.5,0,0),    // do half our translation

Rotation(1,0,0,-pi/2),  // rotate Z so it aligns with world Y

RevoluteJoint,          // place a second joint

Rotation(1,0,0,pi/2),   // rotate back

Translation(0,0,-0.5) {  // translate to center of second block

Block(0.1, 0.1, 1.0) : (color="green", mass=1);

}

}

}

\endcode

To get the links to move, you must have an active gravity, and you must give the links non-zero masses.

\image html tutorial11.jpg Double pendulum, 1 sec

\image html tutorial12.jpg Double pendulum, 2 sec

\image html tutorial13.jpg Double pendulum, 3 sec

The code for the second example looks a little hairy, but it can be cleaned up very easily if we rotate into the joints' coordinate frames, and never rotate out.  I did the above to show that we can use the same blocks as in our previous example.

\section SecPrism Prismatic Joints

The other type of one degree-of-freedom joint is the PrismaticJoint operation, a joint which translates along Z.  Similar to RevoluteJoint, PrismaticJoint takes three optional arguments: an initial position, and two joint limits (again, not implemented as of this writing).  For brevity, I will skip an example of this joint.

\section SecJointParams Optional Joint Params

Like objects, joints have some parameters that are specific to them.

\li \b name="name" : a joint can be given a name, so that it can be referenced later on or accessed from outside the simulation while running.  This is particularly useful in the case of controlling a robot, where the controlled joints are accessed by name.  In the RHex design, the joints are named "hip0" through "hip5", for instance.

\li \b k=spring_constant : linear prismatic and torsional springs are implemented within the CDL.  You can turn any single degree-of-freedom joint into a spring simply by supplying a \b k value.  As of this writing, linear springs are the only types offered (but it wouldn't be hard to add others), and the springs all have fixed set points, so it doesn't make sense to add springs to a controlled joint.  Again, this shouldn't be that hard to modify.

\li \b d=spring_damping : along with the linear spring constant, you can supply a damping term for the spring, supplying a force proportional to the velocity of the spring.  In simulations, damping is extremely important, as numerical errors can lead to instabilities with springs.  A rule of thumb is to always add at least some damping whenever you add a spring constant to a degree-of-freedom.

\section SecFreeBody FreeBody

The last type of degrees-of-freedom you can add to a mechanism is that of FreeBody.  A FreeBody joint consists of three prismatic joints and one spherical joint.  As of writing, it can neither be named nor controlled, and you cannot set spring properties for a FreeBody joint.  It is simply used to disconnect an object from the world, allowing it to move in all six degrees-of-freedom.  FreeBody was used in our very first example, to allow the block to fall.  It is common to start a mobile robot design with FreeBody, as that is what makes the robot "mobile".

*/

/** \page CDLTutorialPreproc Preprocessors for Macro Expansion

As the CDL has no built-in support for functions, conditionals, loops, and other programming constructs, it's suggested to use a preprocessor when writing a sophisticated model.  In a case like the RiSE model, where there are six almost identical legs, it makes a lot of sense to reuse model code when writing a CDL file.  Preprocessors can greatly aid the model developer here.  I have documented two common preprocessors on this page.

\li \ref SecCPP

\li \ref SecM4

\section SecCPP The C Preprocessor

If you are a C programmer, you already know how to use the C Preprocessor.  It's the unix command \e cpp, and it is used to process your source code before compilation.  Calls like \e #include and \e #define are cpp commands.  Since cpp is so easy, it's the fastest way to get up and running with macros in a CDL file.

So, how do you use it?  From the command line, you simply make a call like <em>cpp < infile.cdl > outfile.ocdl</em>.  The output file has a \e .ocdl filename extension, to differentiate the fact that the file has already been processed.  If you fed the original file as a CDL model, it would not parse, thus the need for the processed file.  For developing a model using \e cpp as your preprocessor, simply use it like you would when programming C or C++ code.  Using \e #include to add secondary files to your model, and use \e #define to create macros for later reuse.

A few caveats with the \e cpp \e #define:  You cannot have newline characters within a \e #define, so you must do things like

\code

#define leg_segment \

Block(1.0,0.1,0.1), \

Translation(0.5,0,0),Rotation(0,1,0,pi/2),\

Translation(0.5,0,0),\

Block(1.0,0.1,0.1);

\endcode

The backslash characters tell the preprocessor to continue reading on the next line.  When this is written to an output file, all of those characters end up on one line, making them hard to read.

Also for this same reason, it's difficult to document \e #define's when using \e cpp.  You must use C style comments.

\section SecM4 The M4 Preprocessor

Given the shortcomings of \e cpp, it's desirable to have something less limited and much more powerful.  \e m4 fills these needs very well.  Unlike \e cpp, \e m4 contains logic to do much more advanced constructs.  \e m4 has built-in support for macros, file inclusion, loops, conditionals, plus more.  However, the syntax is unfamiliar and takes some getting use to.

\subsection SecM41-1 Macros with M4

Similar to a \e cpp \e #define, M4 lets you make macros with or without arguments.  The simplest macro would be something like,

\code

define(`mymacro',`expand this')

\endcode

Note that a backtick (`) always starts a string, and a single quote closes it.  Anytime the preprocessor find "mymacro" in the file, it will replace it with "expand this".  Unlike \e cpp, you can have multi-line macros, meaning there are no backslashes, and you can use C++ style comments.  If you are passing arguments to a macro, you can retrieve them with "$1" for the first argument, "$2" for the second argument, and so on.

\code

define(`macro_arguments',`

My first argument: $1.

My second argument: $2.'

macro_arguments(hello,world)

\endcode

Complete documentation for M4 is available at the <a href="http://www.gnu.org/software/m4/manual/">GNU website</a>.

*/

/** \page CDLTutorialRevisions Documentation Revisions

\li \b 2004-04-01 Initial documentation written, \e gch

\li \b 2004-04-04 Info added on preprocessors, how to use CDL, \e gch

*/