/** \page RTIExamplePage Simple example on building models with RTI

In this page we go through a simple example on how to build models

of actuation and sensing using RTI components. Our example is a single

DC motor, operating in voltage mode, coupled to an encoder and a

gearhead, driven by an APEX motor amplifier. The output of the

gearhead is connected to a simulated joint of the DE engine (perhaps a

leg). The command inputs to the motor driver chip are provided

externally.

All of the data processors necessary to build this model are already

provided with the library. We will use ProcDCMotorApex to model the

combination of the DC motor with the APEX drive and the LC filter in

between. The reason these two must be combined in a sinble block is

because there is feedback between the motor and the motor drive due to

the armature current which affects the terminal voltage of the

motor. Attempting to capture as two separate processors would have

resulted in a cycle, which is unsupported by the current RTI

implementation.

In order to avoid cycles, we also need to divide the gearhead into

two processors, ProcGearheadK, which converts position and speed

readings from the simulated joint to position and speed states of the

motor shaft, and ProcGearheadD, which converts the torque output of

the motor shaft to a torque output to the simulated joint. Once again,

attempting to combine these in a single processors would have resulted

in a cycle because the motor model requires shaft speed as an input,

and provides shaft torque as output.

Modeling the encoder is easy, and is handled by ProcEncoder. We

establish the connection to the simulation system with the processors

ProcJointSource and ProcJointSink, which are once again split to avoid

introducing cycles. Finally, external commands are channeled through a

generic ProcSource with proper output channels, and sensor readings

are pushed into a generic ProcSink for external reading.

The following figure illustrates the data flow graph that we seek to

implement using RTI. The figure is organized such that data flows from

the top to the bottom as processors perform their computations.

\image html MotorModelRTI.gif "Figure 1.Dataflow graph for a simple DC motor connected to a simulated joint, diven by external inputs."

\image latex MotorModelRTI.eps "Dataflow graph for a simple DC motor connected to a simulated joint, diven by external inputs."

Note that we also had to add a single ProcSource to supply a constant

battery voltage to the DC motor. An external battery model could be

implemented to change the supply voltage as a function of time as the

motor current drains the battery charge.

The code necessary to create this dataflow graph is quite

straightforward, but a little tedious to put in place. We first need

to define the parameters required by various processors we need.

\code

#define RSIM_TORQUE_CONST 0.0161        // Torque constant ( Nm / A )

#define RSIM_RA           1.34          // Armature resistance (in ohm )

#define RSIM_SPEED_CONST  55.3685       // Speed constant ( rad/s / V )

#define RSIM_ENC_RATIO    2000          // Encoder ratio ( count/rev )

#define RSIM_GEAR_RATIO   (1.0/33.0625) // Gear ratio (output rev / shaft rev)

#define RSIM_GEAR_EFF     (0.9)         // Efficiency of the gear

\endcode

\code

sim::gearhead_param_t gparams = { RSIM_GEAR_RATIO, RSIM_GEAR_EFF };

sim::encoder_param_t encparams = { RSIM_ENC_RATIO, 16 };

sim::dcmotor_param_t mparams = {

RSIM_TORQUE_CONST, RSIM_SPEED_CONST, RSIM_RA

};

sim::apexchip_param_t dparams = { 0.45 };

\endcode

We are now in a position to create all the data processors used in the

dataflow graph.

\code

sim::ProcGearheadK   * gk;

sim::ProcGearheadD   * gd;

sim::ProcEncoder     * enc;

sim::ProcDCMotorApex * motor;

sim::ProcJointSource * jointsrc;

sim::ProcJointSink   * jointsnk;

sim::ProcSink        * extsink;

sim::ProcSource      * extsource;

sim::ProcSource      * battery;

const char *battery_names[]   = { "vsupply" };

const char *extsink_names[]   = { "count", "voltage", "current" };

const char *extsource_names[] = { "dutyfactor" };

// Create the battery processors

battery = new sim::ProcSource( "battery", 1, 1, battery_names );

battery->setValue( 0, 24.0 );

mysink = new sim::ProcSink( "filesink", 1, 5 );

// Locate the simulated CDL joint to be driven

sim::Joint *joint = cdl->findJoint( "motorjoint" );

// Create the motor, gear and encoder models

gk    = new sim::ProcGearheadK( "gearheadk", 0, & gparams );

gd    = new sim::ProcGearheadD( "gearheadd", 0, & gparams );

enc   = new sim::ProcEncoder( "encoder", 0, & encparams );

motor = new sim::ProcDCMotorApex( "dcmotora", 0, &mparams, &dparams );

// Create processor connections to the simulated joint

jointsrc = new sim::ProcJointSource( "jointsource", 0, joint );

jointsnk = new sim::ProcJointSink( "jointsnk", 0, joint );

// Create external command hooks

extsink   = new sim::ProcSink( "externalsink", 0, 3, extsink_names );

extsource = new sim::ProcSource( "externalsource", 0, 1, extsource_names );

extsource->setValue( 0, 0.0 );

\endcode

Finally, we can create all the necessary connection to obtain the dataflow graph of the above figure.

\code

hipsnk->connect( "torque",         gd[i],     "outputtorque" );

msink->connect ( "count",          enc[i],    "count" );

msink->connect ( "voltage",        motor[i],  "voltage" );

msink->connect ( "current",        motor[i],  "current" );

gd->connect    ( "shafttorque",    motor,     "torque" );

enc->connect   ( "position",       gk,        "shaftposition" );

motor->connect ( "dutyfactor",     extsource, "dutyfactor" );

motor->connect ( "vsupply",        battery,   "vsupply" );

motor->connect ( "speed",          gk,        "shaftspeed" );

gk->connect    ( "outputspeed",    jointsrc,  "velocity" );

gk->connect    ( "outputposition", jointsrc,  "position" );

\endcode

It is usually easier to establish the connections starting from the

sinks, going up to the sources. In any case, the completeness of

connections as well as the presence of cycles can be checked with a

simple call to the processor manager.

\code

sim::ProcessorManager::instance()->verify( "RTI:" );

\endcode

When the simulation is run through CDL, data processors are

automatically updated at a rate equal to the integration rate of the

simulation. This means that the oint torque will be appropriately set

at every iteration of the simulation, yielding a fairly accurate

approximation of the motor model. External control commands and sensor

reads must be handled in between calls to advance the simulation time,

usually done from a higher level loop driving the simulation

(seeCDLInterface class documentation).

*/