/** \defgroup RTIReferenceGroup Real-Time Control Interface (RTI) Library

The RiSE Robot Simulation Library provides a collection of components

to facilitate modeling of sensors and actuators connected to various

elements of the simulation such as joints and rigid bodies. These

components are collected under the "Real-Time Interface" library,

which we refer to as RTI throughout this document.

The central concept in RTI is a network of "Data Processors", through

which there is a flow of data as computation for various parts of the

model is performed. In essence, its structure is very similar to

MATLAB's Simulink, or other systems where modeling is done by

connecting together inputs and outputs of functional blocks. The main

differences is that, RTI does not have a graphical user interface for

these connections and it currently only supports \e acyclic connection

graphs.

\image html DataProc.gif "A simple data processor with 3 inputs and 5 outputs. The update method computes output values from the input values."

\image latex DataProc.eps "A simple data processor with 3 inputs and 5 outputs. The update method computes output values from the input values."

In RTI, functional blocks are implemented through classes derived from

the \ref DataProcessor abstract base class. The library provides a

number of useful blocks:

\li <B>Sensor models and data sources:</B>

ProcEncoder,

ProcBatteryPeukert,

ProcGearheadK,

ProcJointSource,

ProcObjectPose,

ProcObjectVelocityG,

ProcObjectVelocityL,

ProcObjectAccelG,

ProcObjectAccelL,

ProcSource,

ProcZerosource,

ProcOnesource

\li <B>Actuator models and data sinks: </B>

ProcDCMotorV,

ProcDCMotorA,

ProcDCMotorApex,

ProcGearheadD,

ProcJointSink,

ProcSink,

ProcFileSink,

ProcNullsink

\li <B>Various utility processors: </B>

ProcAmplifier,

ProcMultiplier,

ProcAdder,

ProcRectifier,

ProcPassthrough,

ProcAdditive,

ProcAccum

If a particular functionality is not covered by these processors, it

is fairly straightforward to create a processor with the required

fnctionality by subclassing DataProcessor. Each of these processors

must implement an "update" method, which reads from the inputs of the

processor and computes the outputs of the processor.

Connections between blocks are created using

DataProcess::connect. Usually, during the initialization of the

simulation, all data processor instances are created and then the

connections are established using calls to this method for all the

created processors. Note that before the simulation can proceed, all

inputs of all data processors have to be connected.

The updates of data processors as well as various bookkeeping tasksare

handled by the ProcessorManager class. Each data processor registers

itself with a singleton instance of the manager upon

creation. ProcessorManager has a single update method, which is

responsible from updating all the data processors in the system in the

\e correct order, determined by the interconnections between different

blocks. RTI guarantees that all inputs of a data processor are up to

date before its update method is called. We also ensure valid

semantics of this data flow scheme by requiring that there are no

cyclic interconnections. RTI issues an error message if an

attempt is made to create a cyclic interconnection graph.

This document also includes a

\ref RTIExamplePage "simple example on how to build models"

using RTI data processors.  */